FLIGHT CONDITION: SUPERSONIC

Question 1 - SAS

- Current Handling Qualities

According to the flying qualities, McDonnell Douglas F4 is a Class IV aircraft. The flight condition makes it a Category B aircraft and it needs to fulfill the following lateral characteristics’ requirements (assuming Level 1 flying condition);

Roll Mode Time Constant: τroll≤1.4 sec

Spiral Mode Time Constant: τspiral ≥28.9 sec

Dutch Roll Mode Damping Ratio: ζDR≥0.08

Dutch Roll Mode Frequency: (ωDR) ≥0.5

The state space is:

∆β∆ρ∆r∆φ=-0.07640-10.01848-8.912-0.792 0.1584 05.7370-0.1841 00100∆β∆ρ∆r∆φ +00.0054575.347 1.069-0.05737-1.59400∆δa∆δr

Characteristic Equation

s4+1.0525s3+5.9577s2+4.7196s+0.0135

Longitudinal Characteristics of Motion

| Characteristics | Roll Mode | Spiral Mode | Dutch Roll Mode |
| --- | --- | --- | --- |
| Eigen Values | λroll= -0.816 | λspiral= -0.00288 | λDR= -0.117±2.40i |
| Damping Ratio | ζroll=1 | ζspiral=1 | ζDR=0.0487 |
| Frequency (rad/s) | ωroll=0.816 | ωspiral=0.00288 | ωDR=2.40 |
| Time Constant (s) | τroll=1.23 | τspiral=348 | τDR=8.56 |

- Open Loop Responses & Parameters

Zoomed in open loop step responses:

It was observed above that the Dutch role made damping had to be increased whole other criteria was fulfilled. Hence a yaw damper is required. Open loop step response from Rudder to Yaw Rate using SISO tool.

- SAS Design

The roll mode and spiral mode time constants are within their respective required ranges. The Dutch roll mode frequency is also within the required range. However, the Dutch roll mode damping ratio does not fulfill the given criteria and is below the required minimum limit. Since here we are required to make coordinated turn hold autopilot and heading autopilot, we need roll mode characteristics to be good meaning there should be relatively fast response and the aircraft should directly go to required bank angle with no overshoot and extra oscillations. Therefore, first a roll damper will be designed to improve the roll mode characteristics and then a yaw damper will be added to improve the Dutch Roll mode characteristics and bring them under the required limits.

Above is the root locus for the roll damper gain Kp. It can be seen that both Dutch roll mode and spiral mode remain insensitive to the roll damper as their poles are very close to zeros. The gain value chosen is Kp = 3. This gives sufficient roll mode characteristics. Next, the yaw damper was added and its new root locus is shown below.

The Dutch roll mode characteristics have improved and now fall under the required limits. The gain value chosen is Kr = -2.88. Below are the new system characteristics after designing the SAS. It can be observed that all flight requirements are now being fulfilled.

| Characteristics | Roll Mode | Spiral Mode | Dutch Roll Mode |
| --- | --- | --- | --- |
| Eigen Values | λroll= -16.9 | λspiral= -0.00446 | λDR= -2.4±0.339i |
| Damping Ratio | ζroll=1 | ζspiral=1 | ζDR=0.99 |
| Frequency (rad/s) | ωroll=16.9 | ωspiral=0.00446 | ωDR=2.43 |
| Time Constant (s) | τroll=0.0593 | τspiral=224 | τDR=0.416 |

- Closed Loop Responses & Parameters

Step Responses of Augmented & Un-augmented systems

Zoomed in open loop step responses:

Yaw Damper Step Response using SISO Tool

Impulse Responses of Augmented & Un-augmented systems

- Simulink Model for SAS

Question 2 – Coordinated Turn Hold Autopilot

- Autopilot Design

The following images show the root loci and pole locations for the gains in the autopilot which are Kbeta and Kphi

The below image shows the zoomed in root locus of the gain Kbeta

The gains selected for the autopilot are Kbeta = -1 & Kphi = 1

- Closed Loop Responses & Parameters

It is observed that sideslip beta remains zero as it should while a unit step response shows that the bank angle phi stabilizes at 1.

- Simulink Model for Coordinated Turn Hold Autopilot

Question 3 – Heading Hold Autopilot

- Autopilot Design

This autopilot is almost the same as coordinated turn hold autopilot. There is just one extra feedback to the same structure as before and that is the heading Psi feedback. Below is the root locus for the heading feedback gain Kpsi. The value chosen for this gain is Kpsi = 10.

- Closed Loop Responses & Parameters

It is observed from the responses that the heading angle stabilizes at 1 due to unity step input. The beta angle remains almost zero while the phi angle first reaches a maximum and when the heading angle is about to be reached then comes back to zero to level off the wings. This can be observed from the moment rate graphs as well where the yaw rate almost remains zero but the roll rate first increases as bank angle increases and then decreases and comes back to zero when the wings have to be levelled once the heading is achieved.

- Simulink Model for Heading Hold Autopilot

MATLAB CODE

% SUPERSONIC - FLIGHT CONDITION % FlIGHT CONDITIONS M = 1.8; % Approach mach number u0 = 1742; % True airspeed Q = 434.5; % Dynamic pressure g = 32.2; % Gravitational acceleration % AIRCRAFT PARAMETERS S = 530; % Wing surface area cbar = 16; % Mean aerodynamic chord b = 38.7; % Wing Span W = 39000; % Mass of aircraft in pounds m = W/g; % Mass of aircraft in slugs Ix = 25000; % Moment of inertia X-Axis Iy = 122200; % Moment of inertia Y-Axis Iz = 139800; % Moment of inertia Z-Axis % Non-Dimensional Stability Derivatives Cl_B = -0.025; Cl_p = -0.2; Cl_r = 0.0400; Cy_B = -0.700; Cy_p = 0.00; Cy_r = 0.0000; Cn_B = 0.0900; Cn_p = 0.00; Cn_r = -0.260; Cn_TB = 0; Cl_delta_a = 0.01500; Cl_delta_r = 0.0030; Cy_delta_a = -0.0100; Cy_delta_r = 0.0500; Cn_delta_a = -0.0009; Cn_delta_r = -0.025; % Dimensional Stability Derivatives Y_v = (Cy_B*Q*S) / (m*u0); % Yv L_v = (Cl_B*Q*S*b) / (Ix*u0); % Lv N_v = (Cn_B*Q*S*b) / (Iz*u0); % Nv Y_B = Y_v * u0; % Y Beta L_B = L_v * u0; % L Beta N_B = N_v * u0; % N Beta Y_p = (Q*S*b*Cy_p) / (2*m*u0); % Yp L_p = (Cl_p*Q*S*(b^2)) / (2*Ix*u0); % Lp N_p = (Cn_p*Q*S*(b^2)) / (2*Iz*u0); % Np Y_r = (Q*S*b*Cy_r) / (2*m*u0); % Yr L_r = (Cl_r*Q*S*(b^2)) / (2*Ix*u0); % Lr N_r = (Cn_r*Q*S*(b^2)) / (2*Iz*u0); % Nr Y_delta_a = (Q*S*Cy_delta_a) / m; % Y delta a L_delta_a = (Q*S*b*Cl_delta_a) / Ix; % L delta a N_delta_a = (Q*S*b*Cn_delta_a) / Iz; % N delta a Y_delta_r = (Q*S*Cy_delta_r) / m; % Y delta r L_delta_r = (Q*S*b*Cl_delta_r) / Ix; % L delta r N_delta_r = (Q*S*b*Cn_delta_r) / Iz; % N delta r % State Space A = [Y_B/u0 Y_p/u0 -(1-(Y_r/u0)) g/u0; L_B L_p L_r 0; N_B N_p N_r 0; 0 1 0 0]; B = [0 Y_delta_r/u0; L_delta_a L_delta_r; N_delta_a N_delta_r; 0 0]; C = [1 0 0 0; 0 1 0 0; 0 0 1 0; 0 0 0 1]; D = [0 0; 0 0; 0 0; 0 0]; states = {'Beta' 'p' 'r' 'Phi'}; inputs = {'Aileron' 'Rudder'}; outputs = {'Sideslip' 'Roll Rate' 'Yaw Rate' 'Bank Angle'}; sys = ss(A,B,C,D,'statename',states,'inputname',inputs,'outputname',outputs) % MIMO sys11 = sys('Sideslip','Aileron'); sys12 = sys('Roll Rate','Aileron'); sys13 = sys('Yaw Rate','Aileron'); sys14 = sys('Bank Angle','Aileron'); sys21 = sys('Sideslip','Rudder'); sys22 = sys('Roll Rate','Rudder'); sys23 = sys('Yaw Rate','Rudder'); sys24 = sys('Bank Angle','Rudder'); % Open Loop Analysis disp('-----------OPEN LOOP ANALYSIS--------------') Characteristic_Equation = poly(A) damp(sys) figure step(sys) openloop_step = stepinfo(sys23) % Closed Loop SAS Analysis disp('-----------CLOSED LOOP SAS ANALYSIS--------------') damp(clsys) figure step(sys) hold on step(clsys) figure impulse(sys) hold on impulse(clsys) clsys23 = clsys('Yaw Rate - r', 'Rudder'); closeloop_step = stepinfo(clsys23) % Coordinated Turn Hold Autopilot disp('-----------Coordinated Turn Hold Autopilot--------------') figure step(BetaToAileron1) hold on step(PhiToAileron1) legend('beta','phi') CoTurnHold = stepinfo(PhiToAileron1) figure impulse(BetaToAileron1) hold on impulse(PhiToAileron2) legend('beta','phi') % Heading Autopilot disp('-----------Heading Autopilot--------------') figure step(HeadingAngle) HeadingHold = stepinfo(HeadingAngle) figure step(BetaToAileron3) hold on step(PhiToAileron3) legend('beta','phi') figure step(RollRate1) hold on step(YawRate1) legend('Roll Rate (p-deg/s)', 'Yaw Rate (r-deg/s)')